MOTORCYCLE ACCIDENT DETECTION APPLICATION FOR ANDROID SMARTPHONES

________________

A Thesis Proposal

Presented to the Faculty of the

Department of Computer Science

University of San Carlos

Cebu City, Philippines

________________

In Partial Fulfillment

of the Requirements for the Degree

BACHELOR OF SCIENCE IN COMPUTER SCIENCE

________________

By

RAFAEL BACUS

CARL PATRICK AGBISIT

March 21, 2014

# Abstract

Recent statistics have shown an increase in motorcycle-related accidents all throughout the Philippines, some of which lead to dead-on-arrival cases. The increased use of smartphones among the population have also been noted by statisticians. These smartphones contain sophisticated hardware, such as accelerometers and gyroscopes, which help deliver dynamic and powerful applications. These integrated hardware can be programmed to detect certain fall conditions, such as during a motorcycle accident. The accelerometer inside a smartphone can be used to detect external forces applied on the smartphone during a fall, and the gyroscope can be used to obtain the orientation of the user of the smartphone. In the event a positive fall is detected, pre-programmed contact numbers will be notified that an accident has occurred; both a message and the location of the user will be given to these contact numbers.  The aim of this study is to provide an application that will notify friends and family of motorcyclists that an accident has happened. The expected result will be that immediate contacts of the user are aware of a life-threatening situation, and can take appropriate action following the events of an accident.

TABLE OF CONTENTS

Abstract	ii

Table of Contents………………………………………………………………………………………………………………….iii

List of Figures	iv

List of Tables	iv

CHAPTER 1 INTRODUCTION	1

1.1 Rationale of the Study	1

1.2 Statement of the Problem	2

1.2.1 General Objective	2

1.2.2 Specific Objectives	2

1.3 Significance of the Study	2

1.4 Scope and Limitation	3

CHAPTER 2 REVIEW OF LITERATURE	5

CHAPTER 3 RESEARCH METHODOLOGY	11

3.1 Research Environment	11

3.2 Research Respondents	11

3.3 Research Instruments & Sources of Data	11

3.4 Research Procedures	11

3.4.1 Gathering of Data	11

3.4.2 Treatment of Data	12

3.5 Design and Implementation	12

3.5.1 Analysis Phase	12

3.5.2 Design Phase	13

3.5.3 Testing Phase and Evaluation	14

3.6 Testing and Evaluation	14

DEFINITION OF TERMS	16

ORGANIZATION OF STUDY	17

REFERENCES	18

APPENDICES	20

Appendix A | Transmittal Letter 1	20

Appendix A | Transmittal Letter 2	21

Appendix B | Research Instruments	22

Appendix C | Timetable of Activities	23

Appendix D | Budget	24

CURRICULUM VITAE	25

# List of Figures

Figure 2.1 Axes of a Smartphone……………………………………………..…………….	7

Figure 2.2 Acceleration-vs-Time Graph…………………………………………..…….....	8

Figure 2.3 Basic Algorithm Diagram…………………………………...……....................	8

# List of Tables

N/A

# CHAPTER 1

# INTRODUCTION

## Rationale of the Study

In the second quarter of 2012, a total of 4210 vehicular crash-related injury cases were reported in the Philippines (ONEISS, 2012). Fifty-one percent of those cases were caused by motorcycle accidents (ONEISS, 2012). Ten of the sixteen, roughly sixty-three percent, dead-on-arrival cases were motorcycle riders. In the first quarter of 2013, the percentage of motorcycle accidents to total vehicular accidents increased to fifty-nine percent, despite the fact there were less vehicular crash-related injury cases reported (ONEISS, 2013). The percentage of dead-on-arrival motorcycle riders increased to sixty-seven percent (ONEISS, 2013). These findings emphasize the rising number of vehicular accidents relating to motorcycles.

The age group for fifty-four percent of total people involved in accidents were ages 20 until 44 (ONEISS, 2013). Meanwhile, as reported by the statistic group Nielsen, young adults between the ages of 25 and 34 have the highest smartphone penetration at eighty-one percent (Nielsen, 2013). The age group that uses smartphones most frequently – 25 to 34 years old – as well as the age group of people involved in vehicular accidents – 20 to 44 – are roughly the same.

Smartphones have increasingly become more sophisticated and powerful with the inclusion of sensors such as accelerometers, gyroscopes, cameras, and microphones (Robin et al., 2010). Additionally, the leading mobile operating system, with fifty-three percent of smartphone users, is the Android operating system (Nielsen, 2013).

The researchers consider utilizing these sensors as well as the open-source Android operating system to develop an application that will detect the occurrence of a vehicular accident, specifically motorcycle accidents. The application will detect external forces on the smartphone using the accelerometer, determine the orientation of the smartphone using the gyroscope, and, if an accident is detected, will send a message to specified contact numbers. The general objective of the research is to be able to notify immediate contacts of the person involved in an accident.

## 1.2 Statement of the Problem

### 1.2.1 General Objective

This study aims to design an application on smartphones using the Android operating system as a post-vehicular accident detection system.

### 1.2.2 Specific Objectives

- Determine the different scenarios of motorcycle accidents

- Implement existing algorithms to detect changes in forces applied to the smartphone and its orientation

- Develop motorcycle accident detection application

- Test and Evaluate application

## 1.3 Significance of the Study

Motor vehicle accidents will always be present in every day commute. Although steps may be taken to reduce the overall percentage of accidents, one can never completely diminish its occurrence. Many of these accidents may also result in injury to the driver of the vehicle or the passengers, leaving them unable to call emergency dispatchers or their loved ones. In addition, the time between the accident and when an immediate contact is notified has significance in determining whether a person will suffer prolonged injuries. Quick response to an accident, or at least notifying an able person such as a family member, can help reduce such injuries. Therefore, this research will benefit:

Motorcyclists The majority of motor vehicle accidents are related to motorcycles. Many of these accidents also resulted in dead-on-arrival cases. A family member that is notified of the accident may be able to transport the driver or passenger of a motorcycle to the nearest hospital if injuries are minimal and if allowed by medical authorities.

Youth Demographic The majority of motorcycle accidents occur with younger age groups; people who are in their mid-20s to early-30s. Although there may be many reasons why measure of central tendency point to this age group, there is also other statistic that states most users of smartphones also revolve around the same ages. A younger motorcyclist is more likely to benefit from an “accident detection” application.

Close Relations Immediate family members and close friends will be notified of any positive detection and they may take appropriate action; whether to contact emergency dispatch, travel to the accident site themselves, or provide any other additional support. Being aware of any accidents can help ease family members.

Computing Field A motorcycle accident application will be made available for future improvements. This can be a starting point for future researchers who want to improve upon the application, such as integrating it with systems for emergency vehicle dispatchers.

## Scope and Limitation

The researchers expect the following topics and preconditions to be within the scope of the research: Statistics show the Android operating system is now the leading operating system for smartphones. A wider audience, those using Android, can benefit from an accident detection application if and when the application is developed. In addition, it would take a much larger effort to test and develop an application for both the Android OS and Apple’s iOS, the operating system for iPhones; effort that is outside the time constraint allowed to the researchers. Therefore, the Android Operating System will be the foundation of this application. The official Android SDK available from the Android Developer website will be used, as well as the standard Android installation on the smartphone.

Motorcyclists are the most prone to motor vehicle accidents, even accidents resulting in fatalities. Therefore, the scope of the research is set around motorcyclists, and not all motor vehicles; meaning cars, trucks, public utility jeepney (PUJs), etc. are excluded from the research. It would require too much effort to test the application within the setting of a car, a truck, a PUJ, etc. The researchers will focus their efforts surrounding the leading reported cases of motor vehicle injuries, which are motorcycle accidents.

In the event of an accident, the application will be able to locate the coordinates of the user and transmit those coordinates to the designated contacts using Google Maps

The researchers assume that the users of this application will be under a data plan, or own adequate “load”, to allow the transmittal of SMS and mobile data. There are a variety of data plans accessible to the public that allow for a fair amount of data to be used. “Globe [Telecom] has provided subscribers various PowerSurf plans with data caps ranging from 20MB up to 1GB depending on their prices” (Noda, 2013).

The researchers assume the following as limitations of the proposed study:

Many owners may still be using smartphones with an older version of the Android Operating System. These phones may or may not have the hardware to support the latest Android OS version. Therefore, the researchers will assume users of the application will have hardware capable of supporting the more recent versions of the Android Operating System, arbitrarily from version 4.0 (“Ice Cream Sandwich”) and beyond.

If the smartphone were to be damaged during an accident and rendered incapable of use, the application will not be able to function as intended.

Areas of the city may or may not be included under the user’s data plan coverage or GPS coverage

During testing, the smartphone will be subjected to physical manipulations with either a dummy or a participant. However, no live testing (using actual motorcycles in motion) will be conducted due in part to the safety of the participant.

# CHAPTER 2

# REVIEW OF LITERATURE

Statistics from the Online National Electronic Injury Surveillance System (ONEISS) suggest an increasing number of motor-vehicle accidents, specifically accidents related to motorcycles, as well as an increase in the number of deaths related to these accidents. Fifty-one percent of vehicular crash-related injury cases reported in the second quarter of 2012 were motorcycle accidents (ONEISS, 2012). By the first quarter of 2013, the percentage rose to fifty-nine percent (ONEISS, 2013). The objective of this research is to be able to notify immediate contacts of the person involved in such accidents, so that they may take appropriate action. Expectedly, there is a high probability that a motorcycle accident will be accompanied with the driver falling from his vehicle. However, related fall-based researches have only focused on real-world falls pertaining to the elderly, and related accident-detection researches mainly talk about automobiles (cars) accidents. This research will try to cover this gap by addressing falls from motorcycle accidents, using smartphone sensors.

One major concept that will be used in this research is fall-detection. Current technology allows smartphones, which have become more powerful with the inclusion of sophisticated sensors such as accelerometers and gyroscopes, to be able to detect fall-conditions. Several related researches have stated that smartphones are in fact capable of detecting falls and analyzing real-time data. Implementing the powerful Android G1 phone for their PerFallD system, Jiangpeng Dai et al. (2010) “propose utilizing mobile phones as a platform for developing pervasive fall detection system.” The aim for their research was to provide an alternative to fall-detection services by utilizing smartphones instead of existing commercial products. Their experimental results exhibit “superior detection performance and power efficiency.” Martín et al. (2012) states, “… smartphones may facilitate acquiring, logging and even processing personal, environmental or social data, captured through virtual and/or in-device physical sensors.” Although their work “focuses on exploring the use of smartphones to perform activity recognition,” it is also closely related to our work in that internal sensors of the smartphone (accelerometer, gyroscope and magnetometer) were used to determine real-time activities. Their results show that with these sensors, they were able to achieve an 88% accuracy. In A Survey on Smartphone-Based Systems for Opportunistic User Context Recognition, it states, “Studies of the range of magnitude and the frequency range of acceleration generated by the body during daily life activities confirm that the capabilities provided by accelerometers embedded in current mobile phones are sufficient for detection of almost the same range of activities as with the current wearable approaches” (Hoseini-Tabatabaei et al., 2013). These three works suggest that smartphone sensors are capable of detecting several types of daily activities. The activity that will be focused on in this research is the act of falling.

Under the concept of fall-detection is the technology of inertial sensors. These are usually sensors that measure the physical motion of an object. The two primary sensor used to determine a fall is the accelerometer and gyroscope. Related research have chosen these two sensors primarily because of their low power cost and more abundant data. “The power consumption of accelerometers is very small compared to other sensing modalities, for instance, the LIS302DL power consumption is less than 1 mW” (Hoseini-Tabatabaei et al., 2013). The gyroscope in mobile phones consume very little power as well: “MEMS-based gyroscopes are considered to have very low power consumption” (Hoseini-Tabatabaei et al., 2013). For perspective, the study of Carroll & Heiser (2010) shows that on average, the HTC Dream smartphone uses only 26.6mW in a suspended state and 161.2mW in an idle state. The Google Nexus One consumes only 24.9mW in a suspended state. This shows that the power consumption of accelerometers of less than 1mW is negligible compared to other activities on the smartphone. Acoustic sensors are not used as the data gathered from it is currently insufficient for the purpose of detecting traffic accidents due to sound clipping from the actual crash as well as background noise, making differentiation between events difficult (White J. et al., 2011). A visual sensor would not be considered; the smartphone would most likely be inside a container, like a pocket or bag, while riding a motorcycle.

Several algorithms can be considered when detecting a fall using accelerometers. However, the work of Kangas et al. has resulted in one of the more accurate algorithms available; “The results indicated that fall detection using a tri-axial accelerometer worn at the waist….is efficient, even with quite simple threshold-based algorithms, with a sensitivity of 97-98% and specificity of 100%” (Kangas et,al, 2008). This research will model the second algorithm tested by Kangas et al., in which the algorithm (START OF FALL + IMPACT + POSTURE) will first monitor when a fall is initially happening, followed by the detection of the impact, and lastly monitor any change in posture.

Most related works, including the group of Kangas et al., are involved with threshold algorithms, algorithms that initiate an event once a certain condition has been met, or a certain threshold value has been recorded. For this research, the threshold value refers to the acceleration of the smartphone during the start of the fall; “The start of the fall was determined as the [graphical] pit before the impact, total sum vector (SVTOT) being equal or lower than 0.6g” (Kangas et al., 2008). The total sum vector equation is as follows:

SV= Ax2+ Ay2+ Az2,

where Ax, Ay, Az is the acceleration in the x-, y-, and z-axis respectively, as seen in Figure 2.1. The sum vector will be the total acceleration experienced by the smartphone. An individual standing would typically experience 1g (g = acceleration of gravity). However, a sum vector value far below 1g, such as 0.6g, is a very good indicator that an individual is falling. Thus, the sum vector will be the first of three factors indicating a fall.

Figure 2.1 Respective axes of the smartphone

The second phase of this algorithm will be the detection of the actual impact. Under an acceleration-versus-time graph, one can see the characteristics of a fall and impact scenario.

Figure 2.2 Bagala et. al, Prototypical acceleration sum vector of a fall. This real-world example illustrates components that are common to many falls.

The trough right before the lowest point in the graph is where threshold value lies and the application will begin to monitor the situation once that value is met. Beginning from the lowest point of the graph to the peak, the sudden spike in the graph indicates the actual impact.

Once the threshold value is crossed, and an impact is identified, if there is a sudden change in posture or orientation of the smartphone, detected by the gyroscope, then it is highly likely that a fall has occurred.

Figure 2.3 simplifies the algorithm the researchers will use:

Figure 2.3 Basic diagram outlining the algorithm used to detect fall conditions

One additional point concerned with the algorithm is the location of the sensor on the body for positive outcomes. Bourke et. al modeled their 2010 research using the Kangas algorithms and mounted their tri-axial accelerometer on the waist. This method proved to be the most effective for the group. As stated by other research groups, “the waist is a popular location for fall-detection system” (Noury et al., 2008a), “as it provides reliable indications of full-body movement, in addition to its ease of acceptance by allowing attachment to an existing waist band” (Mathie et al., 2004). Research focusing on waist-mounted accelerometers are relevant to this study because it is the closest location from the typical pockets where sensors can still be accurate.

An article from the Ride Apart website, written by Wes Siler, features a list of ten common motorcycle accidents, all of which have a high probability of the rider falling from their motorcycle. The most common accident is when a car turns left in front of you. A collision in this scenario would result in the motorcycle driver being hit anywhere along the left side of the motorcycle. Other common accidents are the lowsider and highsider. The lowsider is characterized to occur usually in a turn and caused by the rear tire losing its grip on the road’s surface. The motorcycle will then fall towards the side it is leaning on and will slide to a stop. A highsider is characterized as a violent rotation of the bike around its long axis. The rear wheel typically loses traction, skids, then suddenly gains traction. This motion almost certainly assures the rider will flip head first off their motorcycle. Other common accidents involve entering a corner too quickly, a car changes lane into you, a car hits you from behind, wet roads, and more. All of these common accident scenarios will result in the driver being ejected from his motorcycle even at slow speeds.

Several commercial products have focused on alternative methods of detecting fall conditions. However, these alternatives would not be conducive to the study intended by this research. The fall detection system from Brickhouse (BrickHouseSecurity.com) requires two devices; the portable sensor that a user will carry which will sense whether or not a fall has occurred, and the tele-base unit responsible for communicating an emergency response upon a positive trigger. Other researches that focus on fall detection, such as from Paoli et al., consider a wearable component. In their testing, the sensing node, with no user interface save three buttons, was attached to a belt worn around the waist. A related research in rehabilitation by Patel et al. suggests a full body unit with sensors, embedded within the fabric, which will “extract clinically-relevant information from physiological and movement data” (Patel et al., 2012). The suit would also communicate with the smartphone, displaying data. Although these methods all detect fall-conditions, it would not be beneficial in the context of vehicular accidents. The Brickhouse product, as well as the research done by Paoli et al., would both be more beneficial for home use by the elderly. A full body unit, as tested by the group of Patel et al., would not be feasible as well; it would require the user to don the “suit” before every instance of travel. A more pervasive form of alert, such as through the smartphone alone, would be more consistent with motorcyclists.

Chapter two discusses several of the past researches done that are related to this research. It first discusses the background of the problem in which there are rising motorcycle accidents. The researchers propose an application in which these accidents can be detected by using a smartphone. Fall detection is then described. It involves a three-step algorithm where the smartphone sensors are utilized to determine a fall condition. The algorithm used will model the algorithm used by the group of Kangas et al. Lastly, chapter two touches upon alternative methods used by other researchers that may be less conducive to this research.

# CHAPTER 3

# RESEARCH METHODOLOGY

## 3.1 Research Environment

The locale of the research is limited to the area around the University of San Carlos Talamban campus of Cebu Philippines. The proposed positioning system is limited in support only to the Philippines.

## 3.2 Research Respondents

## The target population of the research is student motorcyclists. The study will use selective random sampling of motorcyclists in and around the university, in order to keep immediate relevance. Slovin’s formula is used to obtain a sample size of 60, considering the population of 150 student motorcyclists within the campus and a margin of error of 10%.

## 3.3 Research Instruments & Sources of Data

The research will use a researcher-made simple semi-open structured interview-questionnaire. Measurement and quantification of relevant data, like orientation and acceleration, will come from the use of the sensors built in the smartphone itself. Other sources of pertinent data include national statistics and physics publications.

Data gathered from the use of built-in sensors may result in semi-biased results and physical simulations of motorcycle crashes cannot be conducted in actuality for safety and legal reasons. For these reasons, a controlled environment is necessary to produce and gather meaningful data. The interview-questionnaire’s reliability will be analyzed using Cronbach Alpha and Normality testing by IBM SPSS.

## 3.4 Research Procedures

### 3.4.1 Gathering of Data

Data from the sample is gathered using an interview-questionnaire, where each element (person) is interviewed only once. This will quantify the public opinion on the possible demand and economic feasibility of the research. Additional data related to this is regarded to past national statistics regarding motorcycle incidents and accidents.

### 3.4.2 Treatment of Data

Data gathered through the interviews will be analyzed using IBM SPSS by way of summation of data and finding medians. The data will then be presented through a tabular form containing the summarized results of the interview-questionnaires.

## 3.5 Design and Implementation

The software development process for the research is an iterative and incremental model. Each component of the application is treated as a separate application and developed using a mini-waterfall model. Revisions are done any time during the development process.

### 3.5.1 Analysis Phase

Existing researches have focused on pedestrian and elderly fall-detection schemes. Fall-detection algorithms vary from research to research. Use of acceleration, g-force value, sudden spikes, and changes of orientation are common techniques utilized by these researches. Of relevance to this research is the use of threshold-based algorithms which begins active assessment of fall only when a certain threshold, like g-force, is broken.

The researcher-made system will employ the use of a modified Kangas et al.’s vector summation algorithm. The application will also observe stricter timeframes to compensate for the discrepancies between pedestrian and motorcycle falls, mainly due to the difference in physical capabilities (acceleration, g-forces). Other parameters are modified for this same reason. This system will also add another element, visual location based on GPS.

Few of these existing systems used techniques to deal with false positives and negatives to promote accuracy as fine-tuning of the systems themselves were done beforehand. Those that did utilized the creation of a ‘critical time period’ wherein the situation is assessed by way of manual termination by user if fall is false. This system also aims for the use of this technique as motorcycle falls are more erratic than pedestrian ones.

This phase will produce a document specifying and assessing the target goal, features, possible problems that may arise, and assumptions.

### 3.5.2 Design Phase

The application model will utilize the Android operating system and its accompanying programming language for application development. Google Maps will be implemented in the system as an external application. The design scheme requires the use of Smartphone sensors, mainly the accelerometer, as well as its telecommunication capabilities.

The sensing algorithm is the algorithm that will be used to detect the occurrence of a fall/crash. It will implement a critical duration of time wherein false positives and negatives are discerned.

The telecommunication algorithm is a sub-program that would send a message and notification to correspondents that were assigned beforehand. This information sent includes the time of incident and location in the form of a URL. This URL will be used to pinpoint a location using Google Maps on the recipient’s device.

The system will employ the system shown in Figure 2.3 to assess the occurrence of an accident. This will be the flow of events that the application is expected to accomplish during a certain run of the application.

The user interface will consist of 3 major screens. The first screen is the main menu from which a user can access the other two screens: the calibration settings, and the phonebook. The calibration screen is responsible for adjusting the program’s algorithms dependent on motorcycle build, usual leaning angle, among other factors. The phonebook is where contacts that would be notified when a crash occurs would be displayed and modified.

As of this phase, prototypes of these components are finished, with their accompanying documentation regarding methods, functionalities, and variables.

3.5.3 Implementation Phase

The system will be composed of five components: user interface, system configuration, monitoring process, data processing algorithm, and notification service. The smartphone itself is a physical constant that, for the purposes of this research, is assumed to be placed in a waist container (small bag, pocket), or a backpack.

The deliverables of this phase are the actual completed components of the programs that may or may not be subject to further revisions dependent on the results of the testing and evaluations.

### 3.5.4 Testing Phase and Evaluation

Debugging will be done on each of the components. Each component is proofed for functionality, and then assembled into an application. The unevaluated compiled application is then installed on the test phone, and will be subject to software and physical testing.

## 3.6 Testing and Evaluation

Onboard sensors will be tested to their default values and telecommunication capabilities are assessed to their neutral capabilities to create a baseline model for software testing. Variables that are relevant to the algorithm are created as to simulate different motorcycle accident circumstances.

Repeated software testing is then conducted by inputting values such that a motorcycle crash is simulated. Such an example is inputting the motorcycle’s current acceleration, the rider’s leaning angle, with physical characteristics like motorcycle build, direction and distance from crash point constant. Afterwards, these data are used in the algorithm to test the detection of the program.

Physical testing is conducted in a controlled environment. In a given motorcycle accident scenario, thin mats are placed appropriately in a spacious room to the possible areas where the smartphone may impact.

A person who has that phone in his person would then throw himself in a way that would emulate that scenario; this can alternatively be done by throwing only the phone itself. The recorded data and its accompanying result are used to check detection success.

Results will be quantified in percentage of accuracy and compared using Weighted Mean. Once tested for accuracy, the final application is assembled and completed. Necessary tests are undertaken again for thoroughness. The application is then planned to be updated steadily for possible commercial use in the future.

# DEFINITION OF TERMS

Accelerometer is a device that measures the acceleration a certain object experiences on a given axis by using a mass at rest as a frame of reference. Some accelerometers can only measure on one axis, while others can measure on at most three axes; the x-axis, y-axis, and z-axis

Algorithm refers to a logical procedure, or a set of instructions, used to solve a problem or perform calculations

Application is a software designed to run or execute on a computer to perform useful tasks. Mobile applications are thus applications designed for use on mobile devices such as smartphones or tablets.

Automobile is a vehicle intended for use on roads, which transport people rather than goods, and typically have four wheels on two axles.

Android Operating System is an operating system, owned by Google Inc., primarily designed for mobile devices such as smartphones and tablets.

G-force (g) aka Acceleration of Gravity is the acceleration of any object moving under the sole influence of gravity, which is 9.81m/s2. A normal value of g exerted upon a person on the surface of the Earth is 1g.

Global Positioning System (GPS) is a navigation system maintained by the United States government that utilizes satellites in medium Earth orbit to provide location and time information to anyone with a GPS receiver.

Gyroscope is a device used to measure or maintain the orientation of an object.

Open Source (software) is the development or production of computer software with its source code licensed and made accessible to the general public for study, alteration, and distribution.

Operating System refers to a collection of essential software that manages computer hardware resources. Several functions of an operating system include memory management, file system management, security, input/output management, etc.

Smartphone is a cellular phone with more advanced capabilities which allow it to perform extended functions such as connecting to the World Wide Web, playing media, capturing photographs, utilizing GPS navigation, having a touchscreen interface, etc.

# ORGANIZATION OF STUDY

Chapter 1 of this research presents the Introduction, which is composed first of the Rationale of the Study stating the purpose for this research; second, the Statement of the Problem, which states the general and specific objectives of the study; third, the Significance of the Study, which outlines the importance and contribution of the study; and lastly, the Scope and Limitation, which summarizes what the output of this research can and cannot do.

Chapter 2 of this study is the Review of Literature, which covers several concepts found in the research. Concepts such as fall detection, power consumption and algorithms used are explained using findings taken from credible references such as statistical authorities and scholarly journals.

Chapter 3 of this study is the Research Methodology, which outlines the procedure the researchers will follow in order to conduct this research. It describes the environment in which testing will be done, what instruments will be used, and how data will be gathered. The methodology also discusses how the application will be designed and implemented

# REFERENCES

Journals

Bagala F., Becker C., Cappello A., Chiari L., Aminian K., Hausdorff J., . . . Klenk J. (2012) Evaluation of Accelermoter-Based Fall Detection Algorithms on Real-World Falls. PLoS ONE Volume 7 Issue 5, 10 pages

Bourke A.K., van de Ven P., Gamble M., O’Connor R., Murphy K., Bogan E., …, Nelson J. (2010). Evaluation of waist-mounted tri-axial accelerometer based fall-detection algorithms during scripted and continuous unscripted activities. Elsevier Journal of Biomechanics Issue 43, 7 pages

Carroll, A., & Heiser, G. (2010, June). An analysis of power consumption in a smartphone. In Proceedings of the 2010 USENIX conference on USENIX annual technical conference (pp. 21-21).

Dai J., Bai X., Yang Z., Shen Z., & Xuan D. (2010). Mobile phone-based pervasive fall detection. Pers Ubiquit Comput 14:633-643, 11 pages

Hoseini-Tabatabaei S. A., Gluhak, A., and Tafazolli, R. (2013). A survey on smartphone-based systems for opportunistic user context recognition. ACM Comput. Surv. 45, 3, Article 27, 51 pages

Kangas M., Knottila A., Lindgren P., Winblad I., Jamsa T. (2008) Comparison of low-complexity fall detection algorithms for body attached accelerometers. Elsevier Gait Posture Volume 28, 7 pages

Martín H., Bernardos A., Iglesias J., Casar J. (2012). Activity logging using lightweight classification techniques in mobile devices. Pers Ubiquit Comput Volume 17, 21 pages

Mathie, M.J., Celler, B.G., Lovell, N.H., Coster, A.C., (2004). Classification of basic daily movements using a triaxial accelerometer. Medical Biological Engineering and Computing 42 (5), 679–687

Noury, N., Galay, A., Pasquier, J., Ballussaud, M., 2008a. Preliminary investigation into the use of Autonomous Fall Detectors. Conference Proceedings of the IEEE Engineering in Medical and Biology Society 2008, 2828–2831

ONEISS (2012). 2nd Quarter (CY 2012) Key Findings. Department of Health Volume 4 Issue 2, 4 pages

ONEISS (2013). 1st Quarter (CY 2013) Key Findings. Department of Health Volume 5 issue 1, 12 pages

Paoli R., Fernandez-Luque F., Domenech G., Martinez F., Zapata J., Ruiz R. (2011). A system for ubiquitous fall monitoring at home via a wireless sensor network and a wearable mote. Elsevier Expert Systems with Applications Volume 39, 10 pages

Patel S., Park H., Bonato P., Chan L., & Rodgers M. (2012). A review of wearable sensors and systems with application in rehabilitation. Journal of Neuroengineering and Rehabilitation, 9:21, 18 pages

White J., Thompson C., Turner H., Dougherty B., Schmidt D. (2011). WreckWatch: utomatic Traffic Accident Detection and Notification with Smartphones

Websites

Brickhouse Alert. 16 March, 2014. From http://www.brickhousealert.com/personal-emergency-medical-alarm.html

Nielsen (2013). SMARTPHONE SWITCH: THREE-FOURTHS OF RECENT ACQUIRERS CHOSE SMARTPHONES. From http://www.nielsen.com/us/en/newswire/2013/smartphone-switch--three-fourths-of-recent-acquirers-chose-smart.html

Noda, Tam. (11 June, 2013). Mobile browsing business growing in the Philippines. Philstar. 25 March, 2014. From http://www.philstar.com/gadgets/2013/06/11/952824/mobile-browsing-business-growing-philippines

Robin L., Baron J. (2010). MEMS & Sensors for Smartphones Report. Yole Développement. From http://www.prlog.org/10792126-mems-sensors-for-smartphones-report.html

Siler, Wes. (August 2013). 10 Common Motorcycle Accidents and How To Avoid Them. Ride Apart. 25 March, 2014. From http://rideapart.com/2013/08/10-common-motorcycle-accidents-and-how-to-avoid-them/

# APPENDICES

## Appendix A | Transmittal Letter 1

Computer Science Department
University of San Carlos
P. del Rosario Street
Cebu City, Philippines 6000

To Whom It May Concern,

Enclosed is a proposed application for detecting motorcycle accidents and delivering emergency messages to preconfigured contact numbers. This thesis proposal was composed in compliance with the requirements for the Degree of Bachelor of Science in Computer Science of the University of San Carlos. It is titled Motorcycle Accident Detection Application for Android Smartphones.

Under Chapter 1 of the report, the Rationale of the Study, Statement of the Problem, General and Specific Objectives, Significance of the Study, and Scope and Limitation are outlined. Chapter 2 covers a number of related researches under the Review of Literature. These past researchers help rationalize the algorithms explained in this report. Chapter 3 discusses the methodology that will be used to complete the research.

The purpose of the proposed application is to provide a way for immediate contacts of the user to be informed that the user has just been in an accident. This allows the contacts to take action they deem necessary. Although within the report there are statistics provided that describe the current numerical figures in motorcycle accidents, it must be clarified that the purpose of this application is not to reduce the number of motorcycle accidents in a given time period. Instead, the primary role of this Android application is to act as a bridge between the user and his close relations in times after a motorcycle accident has occurred, when the user may be injured or unconscious.

Sincerely,

Rafael Bacus	Carl Patrick Agbisit
BS Computer Science Student	BS Computer Science Student

endorsed by adviser: Archival Sebial, MSCS    ___________________

## Appendix A | Transmittal Letter 2

Respondent Address

Dear Respondent,

We are students at the University of San Carlos and are conducting a survey concerning motorcycles and an accident detection application. Recent statistics have shown an increasing percentage of motorcycle accidents in the Philippines, as well as an increase in dead-on-arrival cases. Our group is proposing an application on Android Smartphones that will be able to detect a motorcycle accident by using the onboard sensors to determine if the driver has fallen from his vehicle. Once an accident has been identified, an emergency message will be sent to contact numbers preconfigured by the user of the application. Along with this message, the location of the user will be sent as well.

We are interested to know if such an application would be beneficial to those who drive a motorcycle, or even those who have relations with motorcyclists. Enclosed is the questionnaire. Please feel free to ask us any questions that you may have concerning this survey.

Thank You.

Rafael Bacus	Carl Patrick Agbisit
BS Computer Science Student	BS Computer Science Student

endorsed by adviser: Archival Sebial, MSCS    ___________________

## Appendix B | Research Instruments

The following are likely instruments the researchers will use to complete this research:

- Android Smartphone capable of supporting at least Android Operating System Version 4.0 “Ice Cream Sandwich”

- Eclipse Integrated Development Environment (IDE)

- Android Software Development Kit (SDK)

- Application Development Tools (ADT) plugin for Eclipse IDE

- Statistical Software such as Minitab

- Temporary Questionnaire:

  - “Do you drive a motorcycle?”

  - “How many members in your family drive a motorcycle?”

  - “How many of your friends drive a motorcycle?”

  - “Are you subscribed to a mobile data plan?”

  - “How useful do you think this application would be for motorcyclists?”

  - “How useful do you think this application would be for people who don’t drive motorcycles, but know others that do?”

  - “How useful would this application be for motorcycle passengers?”

  - “If this application was offered at a low cost, would you purchase it?”

## Appendix C | Timetable of Activities

|  | April | May | June | July | August | September | October |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Review Android API | * * |  |  |  |  |  |  |
| Design Classes | * * * * |  |  |  |  |  |  |
| Survey/Data Gathering | * |  |  |  |  |  |  |
| Code Fall Detection Algorithm |  | * * * * | * * |  |  |  |  |
| Code User Interface |  |  | * * | * * * * |  |  |  |
| Test & Evaluate Application |  |  |  |  | * * | * * |  |
| Revise Application |  |  |  |  | * * * * | * * * * |  |
| Finalize Project |  |  |  |  |  |  | * * * |

| * | one week |
| --- | --- |

## Appendix D | Budget

| No. | Item | Cost |
| --- | --- | --- |
| 1 | Used Android Smartphone | 3000 Php |
| 2 | Sports Mat | 2500 Php |
| 3 | Printing Costs | 1500 Php |

# CURRICULUM VITAE

Contact Information

Name: Rafael Buenaventura Bacus

Address: 3rd Floor, Rosedale Place

Dona Rita Village,

Banilad Cebu City,

Philippines 6000

Telephone: 032-236-5167

Cell Phone: 0923-978-2309

Email: rafael.bacus@gmail.com

Personal Information

Birthday: 28 August, 1992

Religion: Christian

Civil Status: Single

Education

University of San Carlos

Bachelor of Science in Computer Science

Tertiary Level (2012 – Present)

Ateneo de Davao University

Bachelor of Science in Computer Science

Tertiary Level (2011 – 2012)

Alpharetta High School

Secondary Level (2007 – 2011)

Hopewell Middle School

Intermediate Level (2004 – 2007)

Manning Oaks Elementary School

Primary level (1999 – 2004)

Technical Skills

Programming Languages: C, Java, HTML, CSS, JavaScript, jQuery, PHP,      		MySQL
Adobe Premiere Pro, Adobe After Effects
Microsoft Office Applications

Work Experience

N/A

Trainings

N/A

Contact Information

Name: Carl Patrick Agbisit

Address: 3rd Street, Paradise Village,

Banilad Cebu City,

Philippines

Cell Phone: 0915-114-9199

Email: carl.agbisit@gmail.com

Personal Information

Birthday: 26 March, 1994

Religion: Agnostic

Civil Status: Single

Education

University of San Carlos

Bachelor of Science in Computer Science

Tertiary Level (2012 – Present)

De La Salle University

Bachelor of Science in Computer System Engineering

Tertiary Level (2010 – 2012)

Caritas Don Bosco School

Secondary Level (2006 – 2010)

Caritas Don Bosco School

Primary Level (2003 – 2006)

Colegio de San Juan de Letran Intramuros

Primary Level (2000 – 2003)

Technical Skills

Programming Languages: C, Java, HTML, CSS

GIMP, Photoshop, 2D art

Microsoft Office Applications

Work Experience

N/A

Trainings

N/A